# CAD Files

Main Body:

https://a360.co/4dpYNwq

https://a360.co/43uJGOG

# Script

For the Aurdino:
// ---------------------------------

// Key definitions

#define BUTTON_KEY1 KEY_ESC       // Escape

#define BUTTON_KEY2 0             // Braille input

#define BUTTON_KEY3 0             // Braille input

#define BUTTON_KEY4 KEY_BACKSPACE // Backspace

#define BUTTON_KEY5 KEY_TAB       // Tab

#define BUTTON_KEY6 0             // Braille input

#define BUTTON_KEY7 0             // Braille input

#define BUTTON_KEY8 KEY_RETURN    // Enter

#define BUTTON_KEY9 KEY_LEFT_SHIFT // Shift (Capitalization)

#define BUTTON_KEY10 0            // Braille input

#define BUTTON_KEY11 0            // Braille input

#define BUTTON_KEY12 32           // ASCII For Space

// Pin definitions (Updated layout)

#define BUTTON_PIN1 10

#define BUTTON_PIN2 16

#define BUTTON_PIN3 14

#define BUTTON_PIN4 15

#define BUTTON_PIN5 6

#define BUTTON_PIN6 7

#define BUTTON_PIN7 8

#define BUTTON_PIN8 9

#define BUTTON_PIN9 2

#define BUTTON_PIN10 3

#define BUTTON_PIN11 4

#define BUTTON_PIN12 5

#include "Keyboard.h"

void failsafe();  // Forward declaration

// Braille dot positions based on button layout

const uint8_t braillePins[6] = {16, 14, 7, 8, 3, 4};

// \0 means nothing, it is there to prevent runtime errors and " " is not viable because it will type a space

// this is according to the braille alphabet so specific combinations will yield nothing

//this is read in a binary fashion, so 000000 is the first term and obviously that is nothing. It increases by binary value

//100000 is the second therm and that is a as you can see in the map

//the first two zeros represent the top row, the middle two zeros represent the middle row, the last two represent the last row

//for example, let say that g is the first row plus the second button in the middle row, the combination for that would be 110100

const char brailleTable[64] = {

'\0','a','\0','c','\0','b','i','f','\0','e','\0','d','\0','h','j','g',

'\0','k','\0','m','\0','l','s','p','u','o','\0','n','\0','r','t','q',

'\0','\0','\0','\0','\0','\0','\0','\0','\0','\0','\0','\0','\0','\0','w','\0',

'\0','v','\0','x','\0','\0','\0','\0','\0','z','\0','y','\0','\0','\0','\0'

};

const unsigned long COMBO_TIMEOUT = 30;

enum BrailleState { IDLE, COMBINING, WAIT_RELEASE };

enum InputMode { MODE_LETTER, MODE_NUMBER, MODE_SYMBOL };

InputMode currentMode = MODE_LETTER;

class button {

public:

const char key;

const uint8_t pin;

button(uint8_t k, uint8_t p) : key(k), pin(p) {}

void press(boolean state) {

if (state == pressed || (millis() - lastPressed <= debounceTime)) return;

lastPressed = millis();

if (key != 0) {

state ? Keyboard.press(key) : Keyboard.release(key);

}

pressed = state;

}

void update() {

press(!digitalRead(pin));

}

private:

const long debounceTime = 20;

unsigned long lastPressed;

boolean pressed = 0;

};

button buttons[] = {

{BUTTON_KEY1, BUTTON_PIN1},

{BUTTON_KEY2, BUTTON_PIN2},

{BUTTON_KEY3, BUTTON_PIN3},

{BUTTON_KEY4, BUTTON_PIN4},

{BUTTON_KEY5, BUTTON_PIN5},

{BUTTON_KEY6, BUTTON_PIN6},

{BUTTON_KEY7, BUTTON_PIN7},

{BUTTON_KEY8, BUTTON_PIN8},

{BUTTON_KEY9, BUTTON_PIN9},

{BUTTON_KEY10, BUTTON_PIN10},

{BUTTON_KEY11, BUTTON_PIN11},

{BUTTON_KEY12, BUTTON_PIN12},

};

const uint8_t NumButtons = sizeof(buttons) / sizeof(button);

const uint8_t ledPin = 17;

void setup() {

Serial.begin(9600);

Keyboard.begin();

pinMode(1, INPUT_PULLUP);

if (!digitalRead(1)) {

failsafe();

}

pinMode(ledPin, OUTPUT);

digitalWrite(ledPin, HIGH);

for (int i = 0; i < NumButtons; i++) {

pinMode(buttons[i].pin, INPUT_PULLUP);

}

}

void loop() {

for (int i = 0; i < NumButtons; i++) {

buttons[i].update();

}

checkBrailleInput();

}

void checkBrailleInput() {

static BrailleState state = IDLE;

static uint8_t storedPattern = 0;

static unsigned long stateStartTime = 0;

uint8_t currentPattern = 0;

for (uint8_t i = 0; i < 6; i++) {

if (!digitalRead(braillePins[i])) {

currentPattern |= (1 << i);

}

}

switch (state) {

case IDLE:

if (currentPattern != 0) {

storedPattern = currentPattern;

stateStartTime = millis();

state = COMBINING;

}

break;

case COMBINING:

if (currentPattern != storedPattern) {

storedPattern = currentPattern;

stateStartTime = millis();

}

if (millis() - stateStartTime >= COMBO_TIMEOUT) {

state = WAIT_RELEASE;

}

break;

case WAIT_RELEASE:

if (currentPattern == 0) {

if (storedPattern != 0 && storedPattern < 64) {

if (storedPattern == 60) {

currentMode = MODE_NUMBER;

}

else if (storedPattern == 12) {

currentMode = MODE_SYMBOL;

}

else {

char outputChar = '\0';

if (currentMode == MODE_NUMBER) {

switch (storedPattern) {

case 1: outputChar = '1'; break;

case 5: outputChar = '2'; break;

case 3: outputChar = '3'; break;

case 11: outputChar = '4'; break;

case 9: outputChar = '5'; break;

case 7: outputChar = '6'; break;

case 15: outputChar = '7'; break;

case 13: outputChar = '8'; break;

case 6: outputChar = '9'; break;

case 14: outputChar = '0'; break;

default: currentMode = MODE_LETTER; break;

}

} else if (currentMode == MODE_SYMBOL) {

switch (storedPattern) {

case 1:  outputChar = '!'; break;

case 3:  outputChar = '@'; break;

case 5:  outputChar = '#'; break;

case 7:  outputChar = '$'; break;

case 9:  outputChar = '%'; break;

case 11: outputChar = '^'; break;

case 13: outputChar = '&'; break;

case 15: outputChar = '*'; break;

case 6:  outputChar = '('; break;

case 14: outputChar = ')'; break;

case 10: outputChar = '-'; break;

case 22: outputChar = '_'; break;

case 18: outputChar = '+'; break;

case 2:  outputChar = '='; break;

default: currentMode = MODE_LETTER; break;

}

}

if (currentMode == MODE_LETTER && outputChar == '\0') {

outputChar = brailleTable[storedPattern];

}

if (outputChar != '\0') {

Keyboard.print(outputChar);

Serial.print("TTS:");

Serial.println(outputChar);

}

if ((currentMode == MODE_NUMBER || currentMode == MODE_SYMBOL) && outputChar != '\0') {

currentMode = MODE_LETTER;

}

}

}

storedPattern = 0;

state = IDLE;

}

break;

}

}

void failsafe() {

for (;;) {}  // Infinite loop

}

Text to Speech GUI:

import tkinter as tk

from tkinter import ttk

import threading

import serial

import pyttsx3

import time

class BrailleTTSApp:

def __init__(self, master):

self.master = master

self.master.title("Braille Keyboard TTS")

self.master.geometry("600x700")

self.engine = pyttsx3.init()

self.engine.setProperty("rate", 250)

self.engine.setProperty("volume", 1.0)

self.running = False

self.thread = None

self.ser = None

# GUI Variables

self.label_var = tk.StringVar(value="Press 'Start Listening' to begin.")

self.connection_var = tk.StringVar(value="Status: Not connected")

# Available COM ports

self.com_ports = self.get_com_ports()

# GUI Layout

ttk.Label(master, text="Braille Keyboard TTS", font=("Arial", 16)).pack(pady=10)

ttk.Label(master, textvariable=self.label_var, wraplength=380, foreground="green").pack(pady=5)

ttk.Label(master, textvariable=self.connection_var, foreground="blue").pack(pady=5)

# COM Port

ttk.Label(master, text="Select COM Port:").pack(pady=5)

self.com_port_var = tk.StringVar()

self.com_port_menu = ttk.Combobox(master, textvariable=self.com_port_var, values=self.com_ports, state="readonly")

self.com_port_menu.pack(pady=5)

# Buttons

self.start_button = ttk.Button(master, text="Start Listening", command=self.start_listening)

self.start_button.pack(pady=10)

self.stop_button = ttk.Button(master, text="Stop Listening", command=self.stop_listening, state=tk.DISABLED)

self.stop_button.pack()

# Volume Control

ttk.Label(master, text="Volume:").pack(pady=5)

self.volume_slider = ttk.Scale(master, from_=0.0, to=1.0, orient="horizontal", command=self.update_volume)

self.volume_slider.set(1.0)

self.volume_slider.pack(pady=5)

# Speed Control

ttk.Label(master, text="Speech Speed (Rate):").pack(pady=5)

self.speed_slider = ttk.Scale(master, from_=50, to_=300, orient="horizontal", command=self.update_rate)

self.speed_slider.set(250)

self.speed_slider.pack(pady=5)

# Log

self.log_frame = ttk.Frame(master)

self.log_frame.pack(side=tk.BOTTOM, fill=tk.BOTH, expand=True, pady=10)

ttk.Label(self.log_frame, text="Log Output:").pack()

self.log_text = tk.Text(self.log_frame, height=8, width=50, wrap=tk.WORD, state=tk.DISABLED)

self.log_text.pack()

# Handle window close

self.master.protocol("WM_DELETE_WINDOW", self.on_close)

def get_com_ports(self):

return ['COM1', 'COM2', 'COM3', 'COM4', 'COM5', 'COM6']

def start_listening(self):

if not self.running:

self.running = True

self.thread = threading.Thread(target=self.listen_serial, daemon=True)

self.thread.start()

self.label_var.set("Listening for TTS input...")

self.connection_var.set("Status: Connecting...")

self.start_button.config(state=tk.DISABLED)

self.stop_button.config(state=tk.NORMAL)

def stop_listening(self):

self.running = False

self.label_var.set("Stopped listening.")

self.connection_var.set("Status: Not connected")

self.start_button.config(state=tk.NORMAL)

self.stop_button.config(state=tk.DISABLED)

def speak(self, text):

self.engine.say(text)

self.engine.runAndWait()

def handle_punctuation(self, text):

punctuation_map = {

'.': " dot ", ',': " comma ", ':': " colon ", ';': " semicolon ",

"'": " apostrophe ", '"': " quote ", '?': " question mark ",

'!': " exclamation mark ", '(': " open parenthesis ", ')': " close parenthesis ",

'-': " dash ", '_': " underscore ", '/': " slash ", '\\': " backslash ",

'=': " equals ", '+': " plus ", '#': " hash ", '$': " dollar ",

'%': " percent ", '&': " and ", '@': " at ",

}

new_text = []

for char in text:

if char.isupper():

new_text.append("capital " + char.lower())

else:

new_text.append(char)

text = ''.join(new_text)

for punct, replacement in punctuation_map.items():

text = text.replace(punct, replacement)

return text

def log(self, message):

def _log():

self.log_text.config(state=tk.NORMAL)

self.log_text.insert(tk.END, message + "\n")

self.log_text.config(state=tk.DISABLED)

self.log_text.yview(tk.END)

self.master.after(0, _log)

def update_rate(self, val):

try:

rate = int(float(val))

# Update only if rate is different

if rate != self.engine.getProperty("rate"):

self.engine.setProperty("rate", rate)

# Restart the engine to apply new rate reliably

self.engine.stop()  # Stop any ongoing speech

# (Optional) Force re-initialization

# self.engine = pyttsx3.init()  # Uncomment only if needed

except ValueError:

print("Invalid rate value received:", val)

def update_volume(self, val):

try:

volume = float(val)

if 0.0 <= volume <= 1.0:

self.engine.setProperty("volume", volume)

except ValueError:

self.log("Invalid volume value")

def listen_serial(self):

while self.running:

try:

selected_port = self.com_port_var.get() or 'COM6'

if self.ser is None or not self.ser.is_open:

try:

self.ser = serial.Serial(selected_port, 9600, timeout=1)

self.master.after(0, lambda: self.connection_var.set(f"Status: Connected on {selected_port}"))

except serial.SerialException:

self.ser = None

self.master.after(0, lambda: self.connection_var.set("Status: Not connected"))

time.sleep(2)

continue

if self.ser.in_waiting:

line = self.ser.readline().decode("utf-8", errors="ignore").strip()

if line.startswith("TTS:") and self.running:

raw_text = line[4:].strip()

if raw_text:

processed = self.handle_punctuation(raw_text)

for char in processed:

if not self.running:

break

self.master.after(0, lambda c=char: self.label_var.set(f"Last Spoken: {c}"))

self.log(f"Spoken: {char}")

self.speak(char)

except serial.SerialException:

self.master.after(0, lambda: self.connection_var.set("Status: Disconnected - Reconnecting..."))

if self.ser:

self.ser.close()

self.ser = None

time.sleep(1)

if self.ser and self.ser.is_open:

self.ser.close()

self.ser = None

self.master.after(0, lambda: self.connection_var.set("Status: Not connected"))

def on_close(self):

self.stop_listening()

self.master.after(500, self.master.destroy)

if __name__ == "__main__":

root = tk.Tk()

app = BrailleTTSApp(root)

root.mainloop()